# Elevator Specifications (please fill in)

| Project name: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. General Spec |  |  |  |  |  |  |  |
| 1.1 |  | Elevator Program |  |  |  |  |  |
| 1.2 |  | Quantity: 数量 |  | 2 sets (duplex)2台 |  |  |  |
| 1.3 |  | Elevator Type:类型 |  | Passenger elevator, 乘客电梯 |  |  |  |
| 1.4 |  | Machine room position: 机房位置 |  | On top of shaft巷道顶部 |  |  |  |
| 1.5 |  | Control Type:控制类型 |  | AC-VVVF with digital encoder交流变频调速带数字编码器 |  |  |  |
| 1.6 |  | Load Capacity: 容量 |  | 630kgs / 8 persons 630公斤/8人 |  |  |  |
| 1.7 |  | Speed:速度 |  | 1.0m/s |  |  |  |
| 1.8 |  | Traveling height:提升高度 |  | 25600mm |  |  |  |
| 1.9 |  | No of floors/stops/entrances:层站 |  | 9 / 9 / 9 |  |  |  |
| 1.10 |  | The floor name 层站名称 |  | G,1,2,3,4,5,6,7,8 |  |  |  |
| 1.11 |  | The depth of pit (mm) 地坑深 |  | 1500mm |  |  |  |
| 1.12 |  | The height of over head净空高 |  | 4500mm |  |  |  |
| 1.13 |  | Shaft size:巷道尺寸 |  | 1750 (W宽) × 1830 (D深) |  |  |  |
| 1.14 |  | Operation操作 |  | Duplex full collective selective control并联集选控制 |  |  |  |
| 2.Technical Parameter :技术参数 |  |  |  |  |  |  |  |
| 2.1 | Drive 驱动 |  |  |  |  | AC-VVVF Inverter driving system 交流变频 |  |
| 2.2 | Control控制 |  |  |  |  | Micro-control / PLC 微机控制/ PLC |  |
| 2.3 | Encoder 编码器 |  |  |  |  | Digitally pulsed type 数字脉冲类型 |  |
| 3. Car design:轿厢设计 |  |  |  |  |  |  |  |
| 3.1 | Car net size (W×D) 轿厢 |  |  |  |  | 1400 (W)×1300 (D) x 2350 (H) |  |
| 3.2 | Destination Indicator 目的地指示器 |  |  |  |  | Digital indicator of floor 数字层数显示 |  |
| 3.3 | Operating Button 操作按扭 |  |  |  |  | Push button 按钮 |  |
| 3.4 | Car rear and side walls 轿厢背部和边墙 |  |  |  |  | Mirror Etching stainless steel finish 镜面蚀刻不锈钢 |  |
|  | Car front wall轿厢前门 |  |  |  |  | Mirror stainless steel finish镜面不锈钢 |  |
| 3.5 | Car door opening type轿厢开门类型 |  |  |  | Two panels center opening 双开门 |  |  |
| 3.6 | Car door opening size开门尺寸 |  |  |  | 800 (W) × 2100 (H) |  |  |
|  | Car door panels轿厢门板 |  |  |  | Mirror Etching stainless steel finish镜面蚀刻不锈钢 |  |  |
| 3.7 | Handrail 扶手 |  |  |  | In 2 side walls.在两个边墙上 |  |  |
| 3.8 | Car Flooring 地板 |  |  |  | PVC |  |  |
| 3.9 | Car operating panel board (COP) 轿厢操纵面板 |  |  |  | Mirror stainless steel finish 镜面不锈钢 |  |  |
| 4. Landing door design:厅门设计 |  |  |  |  |  |  |  |
| 4.1 | Landing door size 厅门尺寸 |  |  |  | 800 (W) × 2100(H) |  |  |
| 4.2 | Landing door opening type厅门开门方式 |  |  |  | 2 panels center opening 双开门 |  |  |
| **Shaft door finish |  |  |  |  |  |  |  |
| 4.4 | At ground floor 第一层 |  | Mirror Etching stainless steel finish with wide (250mm) jamb and transom panel of mirror St. St. 镜面蚀刻不锈钢大门套（250mm）镜面横梁板 |  |  |  |  |
| 4.5 | Other floors其他层 |  | Hairline stainless steel finish & Narrow jamb of Hairline St. St. 不锈钢发纹窄门套发纹 |  |  |  |  |
| 5.General Information |  |  |  |  |  |  |  |
| 5.1 | Power supply 电源 |  | 3phase, 400V, 3-Phase, ±15% 50Hz3项400伏，三项 ±15% 50Hz |  |  |  |  |
| 5.2 | Light supply 照明 |  | 1 phase,220V+-15%AC 50HZ一项20伏+-15%交流50HZ |  |  |  |  |
| 6. Hall push button 按钮 |  |  |  |  |  |  |  |
| 6.1 | At ground floor 第一层 |  | Hall push button in vertical face plate with call button and in transom panel 大厅按钮位于垂直面板上带召唤按钮 |  |  |  |  |
| 6.2 | Typical Floor 标准层 |  | Hall push button in vertical face plate with call button. 大厅按钮位于垂直面板上带召唤按钮 |  |  |  |  |
| 6.3 | Arrival chime 到达钟 |  | On car top 轿厢顶部 |  |  |  |  |
| 7.Safety part |  |  |  |  |  |  |  |
| 7.1 | Safety gear 安全钳 |  | Progressive Safety gear 渐进式安全钳 |  |  |  |  |
| 7.2 | Buffer 缓冲器 |  | Polyurethane buffer 聚氨酯缓冲器 |  |  |  |  |
| 7.3 | Speed governor 调速器 |  | Centrifugal type governor离心调速器 |  |  |  |  |
| 8.Function List of Elevator |  |  |  |  |  |  |  |
| 8.1 Inspection operation检修操作 |  |  |  |  |  |  | 8.2 Emergency operation by electricity |
| 8.3 Automatic operation (Without Operator) 自动操作（没有操作员） |  |  |  |  |  |  | 8.4 Operation by operator 操作员操作 |
| 8.5 Upper automatic door opening上部门自动开启 |  |  |  |  |  |  | 8.6 Automatic door closing delay 自动关门延时 |
| 8.7.Call for opening of this floor 本层召唤开门 |  |  |  |  |  |  | 8.8 Brake checking protection 制动检查保护 |
| 8.9 Full load direct running全载指示运行 |  |  |  |  |  |  | 8.10.Overload stopping 超载停止 |
| 8.11Display of running state运行状态保护 |  |  |  |  |  |  | 8.12 Automatic lighting off 自动断开电源 |
| 8.13 Firefight return 消防返回 |  |  |  |  |  |  | 8.14 Automatic Stopping at failure故障自动停止 |
| 8.15 Disoperation removed移开侵害 |  |  |  |  |  |  | 8.16 Automatic diagnose by button 按钮自动检测 |
| 8.17 Repetitive door closing 重复关门 |  |  |  |  |  |  | 8.18 Safe loop protection 安全回路保护 |
| 8.19 Self study of shaft parameters自我学习巷道参数 |  |  |  |  |  |  | 8.20 Trouble-shooting of elevator 电梯故障解决 |
| 8.21 Emergency lighting应急灯 |  |  |  |  |  |  | 8.22 Three side communication 三边通信 |
| 8.23 Limit protection限制保护 |  |  |  |  |  |  | 8.24 Assessment of immunity 免除测试 |
| 8.25 Door interlock protection 门连锁保护 |  |  |  |  |  |  | 8.26 Floor Display 层显示 |

Escalator Specifications (please fill in)

The angle, width of the stair and the lifting height of the escalator must be provided for offer.

| 1 | Project name: |  |
| --- | --- | --- |
| 2 | Solution Details: |  |
|  | Type |  |
|  | Stop Sequence |  |
|  | Car Group |  |
|  | Traction machine | Permanent magnet synchronous grearless machine |
|  | Quantity of Lift |  |
|  | Rated Load |  |
|  | Rated Speed |  |
|  | Traveling height |  |
|  | Floor/Stop/Door |  |
|  | Car entrances |  |
|  | Operation system |  |
|  | Control System |  |
|  | Door operator type |  |
|  | Car dimensions |  |
|  | Hoistway dimensions |  |
|  | Pit depth |  |
|  | Headroom height |  |
|  | Machine Room |  |
|  | Main floor |  |
|  | Power supply |  |
| 3 | Internal lift car arrangement |  |
|  | Car wall |  |
|  | Car door panels |  |
|  | Car suspended ceiling |  |
|  | Car flooring |  |
|  | Ventilation fan |  |
| 4 | Car and Landing Entrances |  |
|  | Door size |  |
|  | Door type |  |
|  | Landing door finish |  |
|  | Door protection |  |
|  | Door jamb |  |
|  | Sill |  |
| 5 | Signals: |  |
|  | Car operation panel |  |
|  | Calling Board, Landing Indicator |  |
| 6 | Other Requirements |  |

Traction Machine Specifications